THEY ARE NOT UNIT TESTS, CALL THEM SOMETHING ELSE!

EMPR Project Report

Table of Contents

R1- Project Summary	3

R2- Technical Description Of The Group Component Implementation	5

R3 Group Component Testing	8

R4-	14

R5- Individual Component Testing	17

R6- Professional Considerations	21

# R1- Project Summary

Our projects main aim was to use the mBed device along with the Synergy Audio CODEC Module (audio board) to create a device which could record audio through the audio jack, digitise this audio and store it on an SD card. Audio files should then be able to be fetched from the SD card and played back on demand. This process should be controlled via the built in mBed LCD screen and keypad or the PC GUI.

We identified five main sub goals we needed to complete in order to successfully finish this project, these were:

- Record and playback audio via the external audio board at a high sampling rate and resolution

- Write and read files to the SD card using the FatFs file system using the .wav file format

- Use serial communication with the PC in order to implement a PC control interface

- Interface with the MBED keypad and LCD screen to create a menu and control system for the project

- Create a simple and visually pleasing PC GUI that a user of our system could use to control the mBed’s functionality.

At the start of the project we worked out the order in which the different components had to be completed in, and then shared the different tasks to all of the group members. At first our group progress was slow because we struggled to get the audio board to output any audio. In addition, we encountered problems regarding how to save data to the SD card. Due to these problems, our team decided to start working closely with each other in an attempt to solve these issues. Supporting each other with the particularly difficult aspects of the project proved to be invaluable in enabling us to make progress at the start of the project. Once we worked out how to correctly communicate with the audio board and SD card, we found that we were able to work on some tasks individually, which allowed us to achieve the other aspects of the project relatively quickly.

After we completed our project, we found the main issue with our final solution was the low sample rate we had to use (8 kHz), which resulted in a low audio quality. This was as a result of the timing issues we encountered using the mBed board. Unfortunately, this meant that goal number 1 was only partially met. Other than the issue with the sampling rate, all other team goals were completed successfully to create a complete solution, which performed well during testing. Our solution was able to record and digitise audio and then save this audio to the SD card. The user could use either the PC or the keypad interface to navigate to a chosen file and play it back, as well as view a files metadata.

Overall my team worked well together in order to complete nearly all our goals.

My individual specification was to create a PC audio visualiser which uses colour, intensity and motion effects to represent the audio currently being played by the mBed device. I split my individual component into a series of sub goals:

- Devise a method to send audio between the mBed and the PC in real time.

- Ensure there is very little latency in the data transfer process in order to ensure a real time response from the visualiser

- Create a series of mini light shows each showcasing a different visual effect such as colour or motion

- Combine these together into one visually pleasing light show

- One of the light shows should move in time with the beat of any music that is played.

When developing my solution, one of the most complex aspects I needed to design was the beat detector. I found that my beat detector algorithm worked reasonably well with songs that had a strong beat but was quite unreliable if the song’s beat was less clear. Unfortunately, I also ran out of time to combine the different mini light shows together to create one complete one. This meant my final solution was comprised of a browser window with five mini light shows next to each other. Fortunately, I found that this layout still looked relatively visually pleasing.

I used JavaScript and a couple of libraries in order to create my individual project. My solution allowed audio to be input from the audio board into the PC through the PC’s audio jack with very little latency. My program then processes the audio data in order to create 5 mini light shows. These mini light shows allowed me to showcase amplitude and frequency using colour and motion.

Overall my individual project was completed successfully, and I achieved nearly all my main goals.

# R2- Technical Description Of The Group Component Implementation

When designing our project, we concluded the most critical part of our solution would be correctly and efficiently implementing communication with the audio board. Our first decision regarding this was to decide which protocol to send/ receive data to the audio board with. This choice was between: left justified mode, right justified mode and I2S mode. We had been advised that using I2S could prove complex and tricky to implement. However, we were also aware that using I2S would allow our solution to perform better because the I2S library allows for the use of direct memory access (DMA). This had the potential to vastly increase our maximum sampling rate and resolution due to the fast data transfers that DMA allows. However, due to the simplicity of using left justified mode, we decided this would be the best communication protocol for our project due to the limited time we had available. Left-justified mode requires the use of several GPIO lines. A summary of all the wires necessary for communication between the mBed and the audio board in left justified mode is given in figure 1. Unfortunately, we soon discovered that in order to get left justified mode working correctly, a lot of care would be required in order to meet the timing specifications set out in the specification. Due to this, we decided to spend the first few weeks mostly focussed on communicating with the audio board. See figure 2 for our rough timing diagram that we created at this stage of the design.

After a couple of unsuccessful attempts to get the audio board working, likely due to not taking enough care to meet minimum timing requirements, we successfully managed to play a sine wave through the audio board. Unfortunately, we soon encountered a new problem; the GPIO library functions to set and clear the GPIO lines took up nearly all available processing time. Consequently, this left very little time for the CPU to perform other tasks that were needed in this project. At this point it became clear that had we chosen I2S to communicate with the audio board, this problem may have been reduced. The simplest way to solve this problem would have been to use a lower sampling rate and resolution because this would mean less data would need to be transferred in each time period. However, we were already using a very low sampling rate and resolution, therefore, we were conscious that if we decreased this sampling rate further then the audio quality could deteriorate drastically. In an attempt to solve the timing problem, we decided to stop using the CMSIS GPIO library, and instead use direct register manipulation to set and clear GPIO lines. This removed the overhead associated with using the library functions, which had obviously become a big problem when calling these library functions over 8000 times a second (we were aiming for a minimum of an 8kHz sample rate). We found that while we were not able to push the sampling rate to as high as we would like, using direct register manipulation allowed us to increase the sampling rate to 16kHz.

The second piece of hardware that we had to work out how to interface with was the SD card. We decided to use the FatFS library to simplify management of the SD card. This proved to be relatively simple compared to building a file system manager from scratch which would have been highly time consuming.

In order to minimise the number of calls we needed to make to the FatFs file system to read/write data, we decided to use a buffer stored on the mBed built in flash. This buffer allowed us to temporarily store audio that had just been recorded/ was about to be played back. Consequentially, our program did not have to be constantly calling store/ read functions on the SD card, which are relatively slow and therefore would have significantly disrupted the audio record/playback functions. The use of a buffer allowed us to only call FatFs library functions once the buffer was full/ empty.

One of the most challenging design decisions regarding the SD card was how large the buffer should be. We experimented with both a large buffer and a small buffer. When we tried to implement a small buffer, we found that it emptied/ filled up very frequently, which caused a large number of tiny pauses in our audio playback/ recording. These pauses were due to the time it took the mBed to empty/fill up the buffer from the data on the SD card. However, when we tried to use a large buffer, then these pauses became less frequent but longer. Neither of these outcomes was desirable, so we worked hard to try to find the best compromise regarding the size of the buffer, in order to create the smoothest sounding audio possible. Again, DMA would have solved our issues with the time it took to empty/refill the buffer, so it was unfortunate that it was not possible for us to implement DMA in the time available.

A further important decision we needed to make was what audio format to store our audio files in on the SD card. One of the most important factors when choosing a file format was calculating how much overhead would be required when saving/ reading the files. It was important to keep this overhead as low as possible because we were already having timing issues with the audio reads/ writes (as described earlier). As a result, formats like MP3 were out of the question due to the complex algorithms required to encode data in their format. In addition, it was important that we did not use a lossy file format because the audio was already of a low quality. One of our options was to save the data in a fully “raw” form, using our own methods for naming and storing meta data. I felt that while this may have been simpler for us to complete (less to learn compared to learning the standards of an existing file format), it would have reduced compatibility with other systems. Therefore, I felt it was important for us to use an existing simple file format. In the end we decided to use the WAV file format. We chose WAV because it is very simple to implement and has very little overhead; allowing quick read and write speeds. A WAV file is made up of a header (containing information such as file name, size, sampling resolution etc..), followed by the audio data. The audio data must be stored using little-endian (least significant byte first) format, which due to it being unintuitive, was an important fact for us to remember when programming our solution.

A proportion of our time was spent working out the structure of our solution and how we were going to interface the different aspects of our solution together. This was especially important because this was a team project, so we had to ensure that we could integrate the various aspects of our solution that we were working on individually. One of the most challenging aspects was designing and implementing the ability for multiple parts of our system to work together “simultaneously”. The PC interface needed to be able to communicate using serial communication at the same time as the keypad interface, in addition to the actual task the MBED was performing (recording, playback…). It was important for us to ensure that all these aspects could work at the same time without interfering with each other's operation. As the mBed does not have multiple cores, we had to carefully design each component to achieve the effect of an “interactive” system. In order to do this, we used interrupts to control both the audio playback/ recording (highly time sensitive) as well as using interrupts for the keypad interface. The serial communication with the PC GUI ran in the main loop of our program. Figure 2 demonstrates the different layers of our solution and how they interact with each other. Due to the use of two sets of interrupts, any users of our product can continue to use the menu even when audio is being played back. Unfortunately, these interrupts did impact the performance of the different aspects of the system. When a keypad button is pressed during audio playback, the audio playback temporarily stops. This is due to the keypad interrupt interrupting the audio playback interrupt handler. Despite this unintended consequence of using interrupts, we decided that this behaviour was preferable to disabling the keypad during audio playback. An alternative to using interrupts, which would have avoided this problem, would be to use a processor that has multiple cores to run the different aspects of our solution simultaneously.

A further technical aspect that we needed to implement was the use of serial communication between the mBed and the PC. Serial communication was needed for two main purposes. Firstly, the PC needed to communicate with the mBed when the control buttons (play, volume adjust etc...) were pressed on the PC GUI.  Secondly, the mBed needed to communicate with the PC in order to produce an audio intensity graph on the PC interface. In order to communicate via serial on the PC, we decided to use the pyserial library for Python.

We also had to decide what language/ library we would use to create our PC GUI. We decided to use tkinter for Python. Unfortunately, tkinter is not the most powerful graphical library to use and other languages/ libraries are known for offering far more powerful graphical tools (for example VB.net is known for making GUI’s for the windows operating system).  Despite this, we still decided to use tkinter as one of our team members had past experience of using it. A further advantage of using tkinter was that we were already using pyserial for our serial communication, therefore, we concluded that a GUI made in tkinter would be easier to combine with the rest of our solution.

As we approached the project deadline, our group spent some time together to draft possible ways to extend our solution or add extra functionality. We decided we wished to include both hardware and software improvements to our solution. We decided to use a 10 segment LED strip in order to create an intensity graph of the audio being played back in real time. Our first consideration was how we would control it. The simplest way would have been to use GPIO lines to drive each of the 10 LED’s individually, however due to the amount of GPIO lines we were already using for the audio board, we were not able to peruse this route. Our next idea was to use AND and NOT gates to create the logic needed to decode 4 GPIO lines into a one hot representation. It soon became clear that the amount of logic gates and wires was becoming impractical, so we decided to source a pre-built 4-to-16 line decoder. This allowed us to easily create an intensity graph using the LED’s. A possible extension to this aspect which we would have liked to achieve if we had more time, was to use diodes to propagate the currently illuminated LED back to the ones before it, rather than just illuminating one LED at a time.

The other main extension which we attempted was to use the I/O boards built in 7-segment LED to display the real time recording duration. While we were able to interface with the 7-segment LED’s using the I2C interface and display numbers, we encountered a problem when trying to implement the playback time counting into our main solution. After a short period of audio playback the whole board crashed. After some consideration, we concluded this was due to the I2C protocol which we used to communicate with the 7-segment display. This protocol uses acknowledgment bits when each instruction is sent, but unfortunately these were occasionally not being received if the audio interrupts happened to occur at the same time as the acknowledgment bits being sent.

Figure 2- Rough Timing diagram for left justified mode made at the design stage

# R3 Group Component Testing

My first set of tests are requirement and functionality tests, these are used to ensure that each project requirement was met and fully functioning (A1-A4, U1-U4 and D1-D3). My testing uses a mixture of “normal” test data, “boundary” test data and “invalid” test data.
When designing the unit tests, I will ensure that they cover valid inputs (inputs that would be expected in normal operation of the solution), invalid inputs (which should be rejected and any errors this causes should be handled correctly) and finally boundary inputs(where valid and invalid data meet). I also ensured that my tests covered additional functionality and features we added which were not directly required by the specification.

Each test I will perform is given a unique ID (to identify it in discussions), a description of what the test is testing, the input to the test, the expected outcome, if the test completes successfully, the actual outcome observed during testing, evidence of the outcome and then a pass/fail.

I will then perform a set usability tests. Usability tests focus on the user’s interaction with the system and how easily they are able to performs actions with the user interface. Usability tests are crucial because they measure how well everyday users are able interact with our solution, rather than just testing with experts on the design team. These tests are presented differently to the rest of my testing because they were performed very differently. Each task has a cruciality score (to indicate how crucial the task is to the main objectives of our solution). Each task also contains a field for the length of time it took the participant to complete the task as well as a field which records comments about usability directly from the participant.

Requirements Testing

| ID | Test Type | Requirement | Input | Input Evidence | Expected Outcome | Actual Outcome | Outcome Evidence | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Valid | A1/A3- Audio should be recorded and digitised. | Audio line from MP3 player into audio board. Record a few seconds of audio. | N/A | Recorded audio should be played back by the PC from SD card accurately. | Audio file appears on SD card of the audio that has just been recorded |  | Pass |
| 2 | Boundary | A1/A3- Audio should be recorded and digitised. | Audio should be recorded for 1 second (shortest recording time that is practical) |  | Recorded audio should be played back by the PC from SD card accurately. | Audio file appears on SD card of the audio that has just been recorded |  | Pass |
| 3 | Valid | A2/A4- Audio stored in memory should be able to be played back on demand. | A file on the SD card shall be chosen and then headphones plugged into the audio board will listen to the playback |  | Audio should be played back faithfully through audio board | Audio is played back correctly from audio board |  | Pass |
| 4 | Valid | A3/ U2 - User entered file name on recorded audio files | The keypad will be used to enter the name of the file that has been recorded. Name = “abc” |  | A file named “abc” will be present on the SD card | A file named “abc” is present on the SD card |  | Pass |
| 5 | Valid | A4- The user interface allows the user to choose a file to be played back | The user shall choose the file named “abc” from the LCD screen |  | The audio in the file “abc” will be played back when selected | The audio in the file “abc” is played back when selected |  | Pass |
| 6 | Valid | U3- View file list | Press ### to view file list and press ### to scroll through this list |  | The LCD screen will display the current saved files on the mBed and allows the user to scroll through these | The LCD screen displays the current saved files on the mBed and allows the user to scroll through these |  |  |
| 7 | Valid | U3- Return To file selection after playback is complete | An audio file is selected for playback |  | The mBed LCD screen should return to the file list once the audio playback is complete. | The mBed LCF screen remains on its current screen. |  | Fail |
| 8 | Valid | U4- The user can choose a file and view meta data about that file | Press #### on the file list to display the meta data of the file |  | The mBed LCD screen should display size in bytes, playing time, recording rate , and sample size. | The mBed LCD screen displays the size in bytes, playing time, recording rate , and sample size. |  | Pass |
| 9 | Valid | D1- The PC UI can list the files on SD card and their meta data | Click import on PC UI. |  | The PC should list the files currently on the SD card and each files number of bytes and playing time | The PC lists the files currently on the SD card and each files number of bytes and playing time |  | Pass |
| 10 | Valid | D2- The PC should produce an intensity graph where average amplitude is shown as it varies throughout the file. | Click #### |  | The PC should display a graph with axis time and intensity | The PC displays a graph with axis time and intensity |  | Pass |
| 11 | Valid | D3- PC File Manager-Play | Select an audio file and click the play option |  | The mBed should start playing the selected audio file | The mBed starts playing the selected audio file |  | Pass |
| 12 | Valid | D3- PC File Manager-Copy | Select an audio file and click the copy option |  | The mBed should copy the selected file and a new audio file should be created with a new file name and displayed | The mBed copies the selected file and the new audio file has been created with a new file name and displayed |  | Pass |
| 13 | Valid | D3- PC File Manager-Delete | Select an audio file and click the delete option |  | The mBed should delete the selected file from the SD card and it should disappear from the PC file list. | The mBed deletes the selected file from the SD card and it disappears from the PC file list. |  | Pass |
| 14 | Valid | D3- PC File Manager-Volume Adjust | Select an audio file and move the volume slider up |  | The mBed should play its audio at a higher volume | The mBed plays its audio at a higher volume | N/A Demonstrated in team demo | Pass |
| 15 | Valid | D3- PC File Manager-Volume Adjust | Select an audio file and move the volume slider down |  | The mBed should play its audio at a lower volume | The mBed plays its audio at a lower volume | N/A Demonstrated in team demo | Pass |
| 16 | Valid | D3- PC File Manager-Reverse | Select an audio file and click the reverse option |  | The mBed should play its audio at a lower volume | The mBed plays its audio at a lower volume | N/A Demonstrated in team demo | Pass |

Overall I am very pleased with the outcome of the requirements testing, with all but one test passing. The one test which did fail (7), due to the mBed interface not returning to the file list after playback was due to the additional functionality which we added, which allowed the user to multitask during audio playback.

Unit Testing

| ID | Test Type | Description | Input | Input Evidence | Expected Outcome | Actual Outcome | Outcome Evidence | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 17 | Invalid | Enter a file name with length 0 | Enter the file name “” |  | The mBed LCD screen should display an error and ask the user to try again | ???? |  | Fail |
| 18 | Boundary | Scroll past the bottom of the LCD menu interface | Press “#” to navigate down when at the bottom of the menu |  | The menu should wrap around to the top of the menu | The menu wraps around to the top of the menu and displays the first menu option |  | Pass |
| 19 | Boundary | Scroll past the top of the LCD menu interface | Press “*” to navigate up when at the top of the menu |  | The menu should wrap around to the bottom of the menu | The menu wraps around to the bottom of the menu and displays the last menu option |  | Pass |
| 20 | Invalid | Select a PC function without first selecting a file | Click the “play” button without selecting a file |  | The PC GUI displays an error message and asks the user to select an audio file |  |  | Fail |
| 21 | Normal | Test the red recording LED | Choose the “Record” menu option |  | The red LED should be illuminated | The green LED turns on |  | Pass |
| 22 | Normal | Testing the red recording LED at end of recording | Finish an audio recording |  | The red LED turns off | The red LED turns off |  | Pass |
| 23 | Normal | Test the green playback LED | Choose any file to playback |  | The green LED should be illuminated | The green LED turns on |  | Pass |
| 24 | Normal | Testing the green playback LED at end of playback | Allow the mBed to complete the playback of any audio file |  | The green LED turns off | The green LED turns off |  | Pass |
| 25 | Invalid | The user presses the change volume button without changing the volume | Press the change volume button on the PC GUI twice in a row. |  | The PC GUI should display an error message stating that the mBed is already set to the desired volume level | The mBed adjusts the audio level to the same level it is already set to, while giving no user feedback |  | Fail |
| 26 | Valid | Does the LED strip react to the audio amplitude in real time | Playback any audio file |  | LED’s higher up the strip should be illuminated when the audio is louder (use the PC intensity graph to check) | LED’s higher up the strip are be illuminated when the audio is lounder |  | Pass |
| 27 | Normal | Testing multitasking during file playback | When ### file is playing back, choose ### file and view its metadata. |  | The metadata should be displayed without stopping ### playing back. | The metadata should be displayed without stopping ### playing back. |  | Pass |
| 28 | Boundary | Enter a file name with length 1 Tests minimum file name length | Enter the file name “a” |  | The file list should display the file “a” | The file list displays the file “a” |  | Pass |
| 29 | Boundary | Enter a file name with length #### maximum file name length | Enter the file name “###” |  | The file list should display the file “###” | The file list displays the file “###” |  | Pass |

After completing my unit tests I was able to conclude that our solution coped well with valid and boundary data. However, it also demonstrated there was a lack of error handling when the user attempts to perform actions which are not permitted (when invalid data was inputted). In addition to the lack of error handling, the consequences of the user performing these unpermitted actions varied from ########

PLEASED WITH BOUNDARY TESTING

Usability Testing

| ID | Description | Task Cruciality (_/10) | Comments | Time | Name | Task Completed |
| --- | --- | --- | --- | --- | --- | --- |
| 30 | Record some audio and play it back | 10 |  |  |  |  |
| 31 | Use the PC interface to copy a file and then delete the original | 8 |  |  |  |  |
| 32 | Use the mBed interface to choose a file and find the playback time of that file | 5 |  |  |  |  |

# R4-

One of the key decisions to be made when designing my individual component was to decide what language I should program it in. I wanted to choose a language which offered libraries which would make creating graphics easy. I considered languages such as VB.net (allows GUI’s to be created easily but potentially not dynamic graphical effects), python (offers good library support but can be fiddly/ complicated to learn how to use libraries), and JavaScript  (a language which is designed for an interactive web browser and also has good library support). In the end I chose JavaScript with “p5.js” library, as this allows me to create an interactive web browser based environment with p5.js offering plenty of functionality to aid  with moving graphical elements, which would be crucial in creating a light show.

My next decision was how I wished to send audio data in real time from the mBed board to the PC. It was especially important that this happened in real time with minimal latency, because my light show needed to react to the audio live. If the latency was too large then my light show would not look like it was synchronised with the audio playback. I brainstormed two main ways to communicate between the PC and mBed board: serial communication, or a direct audio line between the PC and audio board “out” line. Using serial communication would have been easy on the mBed side because our main group solution was already using serial to transmit audio data to the PC for XXXXXXX. Using serial however had the disadvantage of not being able to transmit audio samples quick enough to use algorithms such as Fast Fourier transform (FFT), which could have limited the range of graphical audio representations I could have created. My other idea of using an audio line instead of serial solved the problem of the audio samples being too transmitted too slow to use FFT. However I needed to find a JavaScript library that would allow me to interface with the audio jack on the PC. In the end I concluded that using an audio line was the superior option, due to the increased flexibility it would give me when designing my light show in addition to the ease of being able to use a simple library to interface with the audio jack.

I then needed to design ideas that would be both visually pleasing and show a good representation of the audio currently being played. I decided I would create a series of “mini light shows”, which could later be combined into one main light show. I decided to do this because I was struggling to design ideas that would be complex enough to reach the specification on their own. When designing the “mini light shows”, I had to ensure they covered as many different aspects of the audio as possible (such as pitch, amplitude and beat).

The five mini light shows I settled on were:

- A simple intensity graph using a colour gradient to represent the audio

- A frequency graph representing the different amplitudes of the frequencies of sound

- Circles which fall from the top to the bottom of the screen with their spawn position being decided by the current volume of the audio

- Circles which flash with the beat of the audio

- A circle with lines representing the current audio level coming out of it

My initial ambition had to use existing beat detection algorithms to create the flashing circles in time with the audio. However after some research, I concluded it would be too time consuming and complex to complete in the time constrains that I was under. In order to still include a beat detector, I designed by own beat detection algorithm. A rough algorithm for my beat detector is sketched out below:

- Set an initial amplitude threshold.

- Keep reading in audio samples. If an audio sample is within a set range of the threshold, the audio sample is classified as a beat.

- If this threshold is exceeded by a given audio sample, the threshold is updated to equal the audio sample.

- Every unit of time (the length of which can be changed), that no beat is detected, reduce the threshold by a set value.

- Goto step 2

The purpose of decreasing the threshold when no beats are detected, was to account for the quieter periods of some songs that are in-between lounder periods. After some initial testing, I found that this algorithm worked reasonably well with songs that had a clear pronounced beat. While this wasn’t my ideal outcome (I had wished to create a beat detector that could cope with all songs), I concluded that I would still include it in my final solution because it would showcase a different aspect of the audio that none of my other mini light shows offered. However had I had more time I would have liked to investigate using more complex beat detection algorithms, which could have worked across a wider range of songs. Although, my research into existing beat detection algorithms highlighted that I would only be able to use some existing algorithms. This is because some existing algorithms would be too slow to be performed in real time and. They are designed instead to analyse pre-recorded audio tracks and then output where the beats in the audio track are once the algorithm has completed.

In order to create the frequency graph, I new I would need to perform FFT on my audio samples. This left me with a decision on if I should attempt to implement the FFT algorithms myself or by using a library. By performing them myself, I would have the advantage of having full flexibility regarding the output of the algorithms, such as the amount of frequency segments to create. This would enable me to tailor the algorithm, so it would perform exactly as I wanted for my light show. However, it soon became apparent that implementing FFT myself would be both time consuming and technical. I also concluded that I would likely not be able to implement it as efficiently as required, as it needed to be able to function in real time to the audio currently playing. After some research I found the library “p5.sound” for JavaScript, which offered me the functionality I required, as well as providing the flexibility that I needed to customise the output for my light show.

Once I had completed each of my individual light shows, I had been hoping to combine them into one complete light show. Unfortunately, I struggled to work out how I would be able to combine them into one in a visually pleasing way, while still maintaining the functionality and unique aspects of each mini light show. Due to this, I decided that I would leave my mini light shows as individual ones. If I had more time, I would have liked to design ways to combine them together. Despite being unable to combine them together, I found I still had to spend time trying to ensure the colours all worked together as well as limiting how much was happening on screen at once.

When designing my light shows, one problem which I was conscious of was lag. Due to this, I concluded it might be important to keep the number of elements on the screen at each moment in time reasonability low. Regrettably, this meant some of my light shows were perhaps not as detailed as they could have been.
In the end however, I found the biggest cause of lag was just printing debugging values to the browser console, and once these were removed, the lag decreased significantly.

DECISIONS OF WHAT EFFECTS TO CHOOSE

WHAT DIDN’T WORK AND WHY- COLOURS CLASHING AND TOO MUCH GOING ON

DESIGN OF SPINNING CIRCLE

EXTENSION DECISIONS

# R5- Individual Component Testing

My testing strategy for my individual component is very similar to the group testing strategy (see section R3 for the detailed plan). There are however some key differences which I have highlighted below.

Unlike the group testing, I will carry out integration testing to ensure that my individual component correctly interfaces with the group component. This is especially important for my task as it was designed as a separate component to the rest of the solution.  It is therefore important that the two components do not interfere with each other’s functionality.

When performing my requirements testing, I will perform both artificial tests (from a tone generator), as well as real world tests (with music). I will use a tone generator because I feel it is important to have constant sounds when doing certain tests in order to be able to fairly compare their results, for example, when comparing the effect of different volume levels on my visualisations. On the other hand, it is also important to use music to test real world applications of my design.

I will also ensure that my tests cover additional functionality and features which I added which were not directly required by the specification.

The final difference is the lack of any usability tests, as my individual component requires no user input so there will be no user centred design features that need testing.

Core Requirements Testing

| ID | Test Type | Requirement | Input | Input Evidence | Expected Outcome | Actual Outcome | Outcome Evidence | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Normal | Colour Based Effect | 1 Khz sine wave played from the PC into audio board via audio jack at 20% volume on PC. |  | The colour of the intensity graph should be mostly green | The colour of the intensity graph was mostly green |  | Pass |
| 2 | Normal | Colour Based Effect | 1 Khz sine wave played from the PC into audio board via audio jack at 80% volume on PC. |  | The colour of the intensity graph should be mostly orange | The colour of the intensity graph was mostly orange |  | Pass |
| 3 | Normal | Colour Based Effect | “Never gonna give you up” played from the PC into audio board via audio jack | N/A | The colour of the amplitude intensity graph will display green to orange depending on the amplitude. When the song is louder the graph will display orange segments. | The colour of the amplitude intensity graph displays green to orange depending on the amplitude. When the song is louder the graph displays orange segments. |  | Pass |
| 4 | Normal | Time Based Effect (Pulsing) | 1 Khz sine wave played from the PC into audio board via audio jack at 20% volume on PC. |  | The blue circle should have a large circumference compared to test 5 | The blue circle has a large circumference compared to test 5 |  | Pass |
| 5 | Normal | Time Based Effect (Pulsing) | 1 Khz sine wave played from the PC into audio board via audio jack at 80% volume on PC. |  | The blue circle should have a small circumference compared to test 4 | The blue circle has a small circumference compared to test 4 |  | Pass |
| 6 | Normal | Time Based Effect (Pulsing) | “Never gonna give you up” played from the PC into audio board via audio jack | N/A | The blue circle should grow and shrink over time depending on the aplitude of the song | The blue circle grows and shrinks over time depending on the amplitude of the song |  | Pass |
| 7 | Normal | Transfer Audio Data In Real Time | Clap into the microphone of the PC. A stopwatch shall be used to measure the time delay between the clap and the intensity graph showing a jump in amplitude. |  | The time delay should be < 1 second |  |  | Pass |

I was very happy that all these tests past successfully. This demonstrates that my solution meets all core requirements of the individual component.

Additional Features Testing

| ID | Test Type | Requirement | Input | Input Evidence | Expected Outcome | Actual Outcome | Outcome Evidence | Pass/Fail |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | Normal | Beat Detection | “Never gonna give you up” played from the PC into audio board via audio jack | N/A | The red circles should flash in time with the beats of the song | The red circles flashes in time with the beats of the song |  | Pass |
| 9 | Boundary | Beat Detection- low volume | “Never gonna give you up” played from the PC into audio board via audio jack at 1% volume |  | The beat detection algorithm should be able to cope with this low volume, and the red circles should flash in time with the beat. |  |  |  |
| 10 | Boundary | Beat Detection- High Volume | “Never gonna give you up” played from the PC into audio board via audio jack at 100% volume |  | The beat detection algorithm should be able to cope with this high volume, and the red circles should flash in time with the beat. | The beat detection algorithm is able to cope with this high volume, and the red circles flash in time with the beat. |  | Pass |
| 11 | Normal | Frequency Graph | 1 Khz sine wave played from the PC into audio board via audio jack. |  | A peak should appear in the first 1/3 of the frequency graph | A peak appears in the first 1/3 of the frequency graph |  | Pass |
| 12 | Normal | Frequency Graph | 3 Khz sine wave played from the PC into audio board via audio jack. |  | A peak should appear in the last 1/3 of the frequency graph | A peak appears in the last 1/3 of the frequency graph |  | Pass |
| 13 | Boundary | Frequency Graph- low frequency | 250 Hz |  | A peak should be seen near the start of the frequency graph | A peak is seen near the start of the frequency graph |  | Pass |
| 14 | Boundary | Frequency Graph- high frequency | 5 kHz |  | A peak should be seen near the end of the frequency graph | No peak is seen |  | Fail |
| 15 | Normal | Amplitude Circle Based Effect | 1 Khz sine wave played from the PC into audio board via audio jack at 20% volume on PC. |  | The lines appearing out of the circle should be much shorter than in test 12 | The lines appearing out of the circle are much shorter than in test 12 |  | Pass |
| 16 | Normal | Amplitude Circle Based Effect | 1 Khz sine wave played from the PC into audio board via audio jack at 80% volume on PC. |  | The lines appearing out of the circle should be much longer than in test 11 | The lines appearing out of the circle are much longer than in test 11 |  | Pass |
| 17 | Normal | “Raining Amplitude Effect” | 1 Khz sine wave played from the PC into audio board via audio jack at 20% volume on PC. |  | Blue circles should appear to the right-hand side of the light show window | Blue circles appear to the right-hand side of the light show window |  | Pass |
| 18 | Normal | “Raining Amplitude Effect” | 1 Khz sine wave played from the PC into audio board via audio jack at 80% volume on PC. |  | Blue circles should appear to the left-hand side of the light show window | Blue circles appear to the left-hand side of the light show window |  | Pass |
| 19 | Invalid | N/A | No audio shall be played. |  | An appropriate error message should be displayed, informing the user that no sound is currently being received. | No error message is displayed. In addition, the light shows visualises NOISE??? |  | Fail |

I was pleased that all tests with normal data input passed. Unfortunately, a problem was noted on two of the boundary tests: the beat detector did not function at a 1% volume level and the frequency graph could not display a peak high enough for 5kHz. In addition, the other test that failed was the “no audio input” test. In this test no error message was displayed, and the light shows didn’t remain motionless. This was likely due to noise in the circuits.

The tests that failed demonstrate a lack of error handling of less common inputs into my visualiser; an area which would need improvement in the future.

Integration Testing

# R6- Professional Considerations

One possible environmental issue with my product is the excess amount of materials that are used in its design. The I/O board of the mBed is very large and is attached to a bulky amount of plastic. This plastic is likely not recyclable. Many existing products that have the same and more functionality as ours (phones, mp3 players etc…), have a much smaller form factor. This means that other products will likely be less costly to dispose of.

In addition, the product is not built to be robust and could easily break (a problem that we encountered multiple times during the design process). This means our product could have a short lifespan, and therefore the consumer may have to buy a replacement quite often. This is clearly a problem because of the environmental cost in disposing of the product, in addition to the environmental cost of the factories producing more products than would be required ifd our solution was more reliable.

nce the product reaches the end of its lifespan then many consumers may choose to dispose  of it in landfill rather than sending it to an electronics recycling company.

The environmental issues with our product are not just at the end of our products lifespan. Factories which produce our product will need to source the raw materials needed to build

This raises the issue of the products lifespan and reliability.